# Q104 Qualification Test Plan

Automotive Operating Temperature Range = 		MSL =

| Supplier Name: |  |  | General Specification: | AEC-Q104 Rev. A |  |
| --- | --- | --- | --- | --- | --- |
| Supplier Code: |  |  | Supplier Wafer Fabrication: |  |  |
| Supplier Part Number: |  |  | Supplier Wafer Test: |  |  |
| Supplier Contact: |  |  | Supplier Assembly Site: |  |  |
| Supplier Family Type: |  |  | Supplier Final Test Site: |  |  |
| Device Description: |  |  | Supplier Reliability Signature: |  |  |
| PPAP Submission Date: |  |  | User Test ID *: |  |  |
| Reason for Qualification: |  |  | User Part Number *: |  |  |
| Prepared by Signature: |  | Date: | User Approval Signature *: |  | Date: |

* Note:  Only required for custom product

| Test | # | Reference | Test Conditions |  | Lots | S.S. | Total | Results Lot/Pass/Fail | Comments: (N/A =Not Applicable) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Group A – Accelerated Environment Stress Tests |  |  |  |  |  |  |  |  |  |
| PC | A1 | JESD22-A113 J-STD-020 | Preconditioning: (Test @ Rm) SMD only; Moisture Preconditioning for THB/HAST, AC/UHST, TC, & PTC; Peak Reflow Temp = |  | Min. MSL = 3 |  |  | MSL = |  |
| THB or HAST | A2 | JESD22-A101 JESD22-A110 | Temperature Humidity Bias: (Test @ Rm/Hot) Highly Accelerated Stress Test: (Test @ Rm/Hot/) |  |  |  |  | of |  |
| AC or UHST or TH | A3 | JESD22-A102 JESD22-A118 or JESD22-A101 | Autoclave: (Test @ Rm) Unbiased Highly Accelerated Stress Test: (Test @ Rm) Temperature Humidity without Bias: (Test @ Rm) |  |  |  |  | of |  |
| TC | A4 | JESD22-A104 | Temperature Cycle: (Test @ Hot) (Sample WBP/WBS post-TC) |  |  |  |  | of |  |
| Test Group A – Accelerated Environment Stress Tests (CONTINUED) |  |  |  |  |  |  |  |  |  |
| PTC or PCT (per Q104, select the appropriate test) | A5A | JESD22-A105 | Power Temperature Cycle: (Test @ Rm/Hot) |  |  |  |  | of |  |
|  | A5B | JESD22-A122 | Power Cycling Test: (Test @ Cold/Hot) |  |  |  |  | Report failures and failure mechanisms |  |
| HTSL | A6 | JESD22-A103 | High Temperature Storage Life: (Test @ Rm/Hot) |  |  |  |  | of |  |
| Test Group B – Accelerated lifetime simulation tests |  |  |  |  |  |  |  |  |  |
| HTOL | B1 | JESD22-A108 | High Temp Operating Life: (Test @ Rm/Cold/Hot) |  |  |  |  | of |  |
| ELFR | B2 | AEC-Q104 Appendix 1 | Early Life Failure Rate: (Test @ Rm) |  |  |  |  | of |  |
| EDR | B3 | AEC-Q100-005 | NVM Endurance & Data Retention Test: (Test @ Rm/Hot) |  |  |  |  | of |  |
| TEST GROUP C – PACKAGE ASSEMBLY INTEGRITY TESTS |  |  |  |  |  |  |  |  |  |
| WBS | C1 | AEC-Q100-001 AEC-Q003 | Wire Bond Shear Test: (Cpk > 1.67) | 30 bonds |  | 5 MCMs Min. | bonds | of |  |
| WBP | C2 | Mil-STD-883, Method 2011 AEC-Q003 | Wire Bond Pull: (Cpk > 1.67); Each bonder used | 30 bonds |  | 5 MCMs Min. | bonds | of |  |
| SD | C3 | J-STD-002 | Solderability MCM External Leads: (>95% coverage) 8hr steam aging prior to testing |  |  |  |  | of |  |
| PD | C4 | JESD22-B100, JESD22-B108 AEC-Q003 | Physical Dimensions: (Cpk > 1.67) |  |  |  |  | of |  |
| SBS | C5 | JESD22-B117 | Solder Ball Shear: (Cpk > 1.67); 5 balls from min. of 10 MCMs |  |  | balls | balls | of |  |
| TEST GROUP C – PACKAGE ASSEMBLY INTEGRITY TESTS (CONTINUED) |  |  |  |  |  |  |  |  |  |
| LI | C6 | JESD22-B105 | Lead Integrity: (No lead cracking or breaking); Through-hole only; 10 leads from each of 5 MCMs |  |  | leads | leads | of |  |
| XRAY | C7 |  | X-ray: |  |  |  |  |  |  |
| AM | C8 |  | Acoustic Microscopy: |  |  |  |  |  |  |
| TEST GROUP D – DIE FABRICATION RELIABILITY TESTS |  |  |  |  |  |  |  |  |  |
| EM | D1 | JEP001 | Electromigration: |  | - | - | - |  | Data Available |
| TDDB | D2 | JEP001 | Time Dependent Dielectric Breakdown: |  | - | - | - |  | Data Available |
| HCI | D3 | JEP001 | Hot Carrier Injection: |  | - | - | - |  | Data Available |
| BTI | D4 | JEP001 | Bias Temperature Instability: |  | - | - | - |  | Data Available |
| SM | D5 | JEP001 | Stress Migration: |  | - | - | - |  | Data Available |
| # TEST GROUP E- ELECTRICAL VERIFICATION |  |  |  |  |  |  |  |  |  |
| TEST | E1 | User/Supplier Specification | Pre and Post Stress Electrical Test: |  | All | All | All | of |  |
| HBM | E2 | AEC-Q100-002 ANSI/ESDA/JEDEC JS-001 | Electrostatic Discharge, Human Body Model: (Test @ Rm/Hot); (1KV / Class 1C or better) |  |  | See test method |  | of ESD Level = |  |
| CDM | E3 | AEC-Q100-011 ANSI/ESDA/JEDEC JS-002 | Electrostatic Discharge, Charged Device Model: (Test @ Rm/Hot); (TC 250 / Class C1 or better) |  |  | See test method |  | of ESD Level = |  |
| LU | E4 | AEC-Q100-004 JESD78 | Latch-Up: (Test @ Rm/Hot) |  |  | See test method |  | of |  |
| # TEST GROUP E- ELECTRICAL VERIFICATION (CONTINUED) |  |  |  |  |  |  |  |  |  |
| ED | E5 | AEC-Q100-009 AEC-Q003 | Electrical Distributions: (Test @ Rm/Hot/Cold) (where applicable, Cpk >1.67) |  |  |  |  | of |  |
| FG | E6 | AEC-Q100-007 | Fault Grading: |  | - | - | - | Fault Grade |  |
| CHAR | E7 | AEC-Q003 | Characterization: (Test @ Rm/Hot/Cold) |  | - | - | - |  |  |
| EMC | E8 | SAE J1752/3 | Electromagnetic Compatibility (Radiated Emissions) |  |  |  |  |  |  |
| SER | E9 | JESD89-1 JESD89-2 JESD89-3 | Soft Error Rate |  |  |  |  |  |  |
| LF | E10 | AEC-Q005 | Lead (Pb) Free: (see AEC-Q005) |  | - | - | - |  |  |
| TEST GROUP F – DEFECT SCREENING TESTS |  |  |  |  |  |  |  |  |  |
| PAT | F1 | AEC-Q001 | Process Average Testing: (see AEC-Q001) |  | All | All | All | Reject units outside Avg. |  |
| SBA | F2 | AEC-Q002 | Statistical Bin/Yield Analysis: (see AEC-Q002) |  | All | All | All | Reject units outside criteria |  |
| TEST GROUP G – CAVITY MODULE INTEGRITY TESTS |  |  |  |  |  |  |  |  |  |
| MS | G1 | JESD22-B110 | Mechanical Shock: (Test @ Rm) |  |  |  |  | of |  |
| VFV | G2 | JESD22-B103 | Variable Frequency Vibration: (Test @ Rm) |  |  |  |  | of |  |
| CA | G3 | MIL-STD-883 Method 2001 | Constant Acceleration: (Test @ Rm) |  |  |  |  | of |  |
| GFL | G4 | MIL-STD-883 Method 1014 | Gross and Fine Leak: |  |  |  |  | of |  |
| TEST GROUP G – CAVITY MODULE INTEGRITY TESTS (CONTINUED) |  |  |  |  |  |  |  |  |  |
| DROP | G5 | JESD22-B110 | Mechanical Shock Cavity Drop Test: (Test @ Rm) |  |  |  |  | of |  |
| LT | G6 | MIL-STD-883 Method 2024 | Lid Torque: |  |  |  |  | of |  |
| DS | G7 | MIL-STD-883 Method 2019 | Die Shear: |  |  |  |  | of |  |
| IWV | G8 | MIL-STD-883 Method 1018 | Internal Water Vapor: |  |  |  |  | of |  |
| TEST GROUP H – MODULE SPECIFIC TESTS |  |  |  |  |  |  |  |  |  |
| BLR | H1 | IPC-9701 AEC-Q007 | Board Level Reliability: TC = |  |  |  |  | of to cycles with no events |  |
| LTSL | H2 | JESD22-A119 | Low Temperature Storage Life: (Test @ Cold/Hot/Rm) |  |  |  |  | of |  |
| STEP | H3 | ISO 16750-4 | Start Up and Temperature Steps: |  |  |  |  | of |  |
| MCM DROP | H4A | JESD22-B110 | MCM Drop Test A: 5 drops each per ±X, ±Y, ±Z |  |  |  |  | of | Report available upon request |
|  | H4B | JESD22-B111 | MCM Drop Test B: Drop in Z direction. Minimum 10 drops, characterize to 100 drops. |  | 1 | 24 | 24 | of to drops with no events | Report available upon request |
| DPA | H5 | MIL-STD-1580 | Destructive Physical Analysis: |  |  |  |  |  |  |
| XRAY | H6 |  | X-ray: |  |  |  |  |  |  |
| AM | H7 |  | Acoustic Microscopy: |  |  |  |  |  |  |

Additional rows can be added as needed for MCM stresses based on other standards, such as AEC-Q102 specific testing.

| Test | # | Reference | Test Conditions | Lots | S.S. | Total | Results Lot/Pass/Fail | Comments: (N/A =Not Applicable) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ADDITIONAL STRESSES |  |  |  |  |  |  |  |  |